Windmills and Rainfall

submitted to the

Government of Maharashtra

July, 2004

Expert Committee Report

Windmills and Rainfall

Submitted to the

Government of Maharashtra

July, 2004

Executive Summary

This report contains the results of the scientific study conducted by the committee on the windmill – rainfall relationship over the Satara region. The committee visited the wind farms at Satara and heard public representations about this issue.  The history of wind farm development in the area under consideration has been looked at in detail and the chronological developments studied.  The international, and national developments in the field of wind power development have been highlighted.  All reported environmental impacts of wind energy conversion systems worldwide have been considered.  The climatology of the region has been very carefully examined.  The parameters such as mean annual and summer monsoon rainfall patterns for Maharashtra on a district wise basis have been reported based on the data available with the India Meteorological Department.  The variations of rainfall across the Sahyadri range of hills (Ratnagiri- Satara region) are reported. The 100 year rainfall data from the region of interest and the neighboring areas were analyzed and interpreted.  The physical dimensions of the monsoon system vis a vis the wind farm sizes were considered. The studies on energy aspects, aerodynamic aspects, wind patterns in the region etc. were also conducted.  The behavior of wind flow over the hills was modeled in order to study the effect of obstruction caused. The committee, based on these scientific facts unanimously came to the following conclusions and recommendations.

- The observed decrease in rainfall over the region during the last 5 years is part of large-scale natural variability.  There have been similar instances in the past 100 years.

- Summer monsoon westerly flow extends up to about 5-6 Km vertically. The effect of these wind mills is however limited to roughly 100 m vertically and 200 m horizontally.

- Most of the time the base of low-level clouds is 1500 m above mean sea level and have gigantic dimensions, and the windmills cannot physically disturb such clouds.

- The energy exploited by the windmills is an extremely small fraction of the total wind energy that crosses the wind farm site.

- The windmills, while rotating, cause a small reduction in the wind speeds which is made up through mixing with surrounding wind stream in a very short distance behind it.

- Wind farms by their very design do not act as a solid obstruction. Therefore, they do not induce enough vertical velocities to result in any appreciable change in precipitation.

- The flow computations past the windmill blades show no perceptible change in temperature field. Hence, there would be no change in cloud morphology.

- The Committee thus has come to a unanimous conclusion that the windmills have no effect on rainfall.

- The Committee feels that the present controversy has arisen due to lack of proper understanding and awareness among the general public, leading to an apparent linkage between the development of the wind farms and the decrease in rainfall over the region.  Therefore, it is recommended that efforts may be made to educate the masses on the environmental friendliness of the wind energy conversion systems.

- As there seems to be an inadequate appreciation in the public mind about the rainfall variability and the robustness of the monsoon phenomenon, it is desirable that the concerned agencies, universities, NGOs, etc. may be assigned the task of education and training, necessarily in the local language.

- All efforts may be made to contain the effects of such controversies on the development of environment-friendly technologies in the state taking into account concerns of local people.

Contents

      - Introduction	5

Background 	5

Documents placed before the Committee	9

      - Wind power technology and wind farm projects in the region	10

      - Climatology of the region	17

      - Approach	22

Site visit	22

Interaction with the public	22

Review of similar issues worldwide	22

Methodology	23

      - Do windmills affect rainfall?	25

Introduction	25

Aerodynamic aspects in large wind farms	25

The scales of the problem	29

Wind patterns in the region	30

Summer monsoon characteristics	31

Weather windmills can affect the clouds by churning them?	35

Effect on temperature	38

Obstruction to the wind flow	38

Scale analysis	39

      - Observed rainfall variations	40

Interannual rainfall variations	40

Drought susceptibility	41

Comparison with neighbouring regions	41

Conclusions and Recommendations	56

Conclusions	56

Recommendations	57

Acknowledgements	58

List of Annexures	59

Appendix I       :	Government of Maharashtra order appointing the	60 committee and grant of extension.

Appendix II	:   News paper advertisement for public response	64

Appendix III	:  Summary of public response	66

Chapter 1

INTRODUCTION

# Background

In the current scenario of growing population, rapidly changing life styles and phenomenal growth in energy demands, finding renewable and environment-friendly sources of energy is being increasingly felt critical to sustainable development.  In this context, around the world, wind is gaining recognition as one of the cleanest sources of power. The premise is simple; there are many areas experiencing persistent high-speed winds, whose energy can be harvested in an optimal manner using appropriate technology.

Wind has been the fastest growing energy technology in the world for the past decade. In 1999, the world wind industry installed a record amount of new utility-scale wind generation equipment, more than 3,900 megawatts (MW), representing investments totaling nearly $4 billion. Total wind installations have increased with an average of 40 percent annually from 1995-1999 to nearly 14,000 MW worldwide. Much of that growth is due to cost reductions and progressive government policies.  The current technology allows a very efficient and optimal exploitation of energy at economically very attractive terms.

Wind energy has only recently gained a foothold in India, and is now                 well-recognized as a viable alternative source of energy in many states.  In the state of Maharashtra, the Maharashtra Energy Development Authority (MEDA) has been spearheading the development of non-conventional energy resources over the past several years.   After making detailed survey of suitable windy sites, MEDA has put up wind energy plants at several places in Maharashtra, taking up a leading position in the development of wind energy projects in India. Around            800 windmills have been erected at Thoseghar, Chalkewadi and Vankuswade, which produce 300 MW power.  More details of these initiatives are provided            in Chapter 2.

Developing local sources to meet our energy needs means that we import less fuel from other states, regions, and nations, thus our energy funds are plowed back into the local economy. Wind energy can also help diversify the economies of rural communities and can generate jobs.  Wind energy is a hedge for the future as our traditional fossil fuels become more scarce and public policies assign environmental costs to sources of pollution.

While there is a general appreciation of the fact that wind energy is a clean source of power, and is also economical in the long term, there has been some public concern about their possible role in local climatic anomalies.  In particular, the occurrence of drought conditions in some of the areas where windmills have been erected has led to the spread of the belief that the windmills have something to do with the decrease in rainfall.  A number of articles appeared in the newspapers highlighting the claims that the windmills are the main cause of reduced rainfall. There have indeed been sincere attempts to assuage these fears through scientific arguments by some individuals/eminent scientists, but they did not have the desired effect.  In the year 2002, MEDA has appointed a Committee headed by Prof. M.G. Takawale to study the effects of windmills on the environment with particular thrust on rainfall.  This Committee, after visiting the sites at Thoseghar, Chalkewadi and Vankuswad and detailed deliberations, has drawn the following conclusions.

- The process of monsoon rain is dependent on the variation of atmospheric condition in the Indian sub-continent. Similarly it is dependent on the percentage of moisture formed on the Indian Ocean and Arabian Sea.

- The area occupied by the windmills is negligible compared to the total land area. Hence they cannot affect the monsoon rains.

- If the clouds are reflected or spun then there should have been more rains in Koyna valley. But the rainfall in Koyna has also decreased.

- There is reduction in rainfall in the Western ghats in the last 3-4 years. Particularly the rainfall in the Satara-Kolhapur region has been reduced. Though no windmills have been installed on the Kolhapur plateau still the amount of rainfall has reduced.

- There is no scientific evidence to show that windmills reduce rainfall. People must be counseled to remove this apprehension. Special awareness programs need to be organized.

However, with the continuation of drought situation in the areas in close proximity of the wind farms, the public concerns about the role of windmills in rainfall anomalies have increased.  Taking cognizance of this situation, the Government of Maharashtra, vide Government Resolution No. Pawan 2004/CR-1820/NRG-7 dated 12 May 2004 (please see Appendix I), appointed a committee consisting of the following experts:

- Dr. G.B. Pant	-	Chairman
Director,
Indian Institute of Tropical Meteorology,
Pune - 411 008.

- Dr. M. Rajeevan	-	Member
Director,
India Meteorological Department,
Pune - 411 005.

- Shri M.P. Ramesh	-	Member
Executive Director,
Centre for Wind Energy Technology,
Chennai - 601 103

- Prof. K. Sudhakar	-	Member
Aerospace Engineering Department,
I.I.T, Mumbai - 400 076

- Shri Mahesh Vipradas	-	Member
Fellow,
The Energy & Resources Institute (TERI),
New Delhi - 110 003

- Prof. A.D. Tillu	-	Member
Visiting Professor,
Department of Atmospheric & Space Sciences,
University of Pune, 
Pune - 411 007

- Dr. S.V. Singh	-	Member
Director,
National Centre for Medium Range Weather Forecasting, 
New Delhi - 110 003

The pertinent terms of reference of the committee are as follows:

- The committee shall give an opportunity to the concerned parties to present their opinion on "Whether windmills installed on Sahyadri plateau is the cause for scanty rainfall".

- Committee shall study various aspects of the above question in detail. If required, committee may visit the project sites in the state, and collect the necessary information of wind power projects in the other states as well as in other countries and give its findings based on such information.

- Committee shall submit its reports on the effect of windmills on rainfall at present and their likely effect in future. In case the Committee comes to the conclusion that windmills are affecting rainfall at present or may affect rainfall in future, they may suggest remedial action to Government.

- Committee shall submit report in two months from the date of its constitution.

- Director General, MEDA, Pune, shall be convener of the Committee.

The Committee initiated its proceedings at the Indian Institute of Tropical Meteorology, Pune. The first meeting of the committee took place on 3rd and       4th June 2004 which included discussion on various aspects of the problem. Detailed presentations were made by expert members on topics such as

- present position of windmills in Maharashtra, other places in the country

- available information about windmills in many other countries where windmills are extensively used

- rainfall climatology and variability of the region under consideration

- cloud morphology

- wind energy extraction

The committee visited the wind farm site in Satara District on 3rd June and had a detailed survey of the installed windmills. Committee also invited concerned people from the public by issuing an open newspaper advertisement to which the response was overwhelming (Appendix-II). Different groups and individuals were given a chance to make presentation on their point of view individually and also collected the written submissions (summary enclosed in Appendix-III).

The reports of the earlier committees appointed by the Maharashtra Energy Development Agency (MEDA) were also brought to the notice of the committee. The controversies and comments which have appeared in the local press during the last few years were also placed before the committee.

The committee had extensive deliberations, on various aspects of the issue. The deliberation further continued through e-mail. Committee members also submitted inputs to the Chairman on their specific area of expertise. The second meeting of the committee was held at Pune on 30th June and 1st July, 2004 to finalize the report and frame the recommendations. The report was vetted, approved and signed by the members present on the last meeting on 23 July, 2004 at IITM.

Considering all the data and studying all aspects of the present issue, the Committee came to a unanimous conclusion that the windmills have no effect on rainfall either at the present time or in the future. The details of the basis are elaborated in the report.

# Documents placed before the Committee

The Committee perused the following documents at various stages of its deliberations:

- Report of the Prof. M.G. Takawale Committee constituted by MEDA in 2002 for a similar purpose.

- Copies of many relevant news paper reports on the subject in the media during the last few years.

- Documents pertaining to technical details of Sahyadri windmill projects.

Chapter 2

WIND POWER TECHNOLOGY AND WIND FARM PROJECTS IN THE REGION

Throughout history, people have harnessed the wind in many ways. Over  5,000 years ago, the ancient Egyptians used wind power to sail their ships on the Nile River. Later, people built windmills to grind their grain. The earliest known windmills were in Persia (Iran). Windmills were also believed to have been extensively used for irrigating lands in Babylonian civilizations as well. These early windmills looked like large paddle wheels. Centuries later, the people of Holland improved the basic design of the windmill. They gave it propeller-type blades made of fabric sails and invented ways for it to change direction so that it could continually face the wind. Windmills helped Holland become one of the world’s most industrialized countries by the 17th century.  American colonists used windmills to grind wheat and corn, pump water, and cut wood. As late as the 1920s, Americans used small windmills to generate electricity in rural areas without electric service.

Initially the windmills were developed and extensively used in the western countries. A major part of the research on windmill technology was carried out in European continent. The older models of windmills are still available and are also preserved in countries like Holland, Denmark and Germany. That is the reason they are called the countries of windmills.

Wind power generation for grid integration was started as early as in the 1940’s when Smith-Putnam designed a 1.25 MW wind turbine and installed at Grand pa’s knob in the United States.  This development met serious mechanical problems and developments of grid connected wind turbines started again in late fifties and early sixties.  In order to gain an insight into the wind turbine design, Danish engineers fully instrumented a 200 kW wind mill which later came to be known as Gedser windmill (Gedser is the name of a town in Denmark).  This machine virtually was the foundation for all future designs of wind turbines in Europe.  Early machines were rated at 50 to 60 kW in unit sizes.

The development of wind energy in real sense started only after the Second World War. Since then there have been rapid advancements in the design of windmills to generate mechanical as well as electrical power. The oil shortages of the 1970s changed the energy picture all over the world. It created an environment more open to alternative energy scenario, paving the way for the re-entry of the windmill to generate electricity. Today, the windmill's modern equivalent, a wind turbine, can efficiently use the wind's energy to generate electricity.

Realising the importance of renewable sources of energy, the Government                     of India embarked on many programmes under the aegis of the Ministry of                  Non-conventional Energy Sources to document and evaluate the alternate power sources. An extensive survey of Wind Power Potential was successfully completed by the Indian Institute of Tropical Meteorology, Field Unit at Bangalore under the dynamic leadership of Miss Anna Mani. The data and documents produced by this project has provided basic information for initiation of Wind Power Project in the country. An offshoot of this project is a full fledged research and consultancy institute under the Ministry of Non-conventional Energy Sources called the Centre for Wind Energy Technology at Chennai.

In the beginning the capacities of Wind Turbines were limited to 100 KW  only. Successively it kept on rising and till today the efforts are being put into to develop wind turbines of 4000 to 5000 kW capacities.  The world wide trend of increase of capacities is given in the following table (Table 2.1).

Table 2.1 :	Growth of unit sizes with respect to time

| Year | Capacity (kW) |
| --- | --- |
| up to 1980 | 50 |
| 1980 to 1990 | 50 to 200 |
| 1990 to 1995 | 200 to 500 |
| 1995 to 2000 | 500 to 2000 |
| 2000 till today | 2000 to 5000 |

However in India we have wind turbines of maximum 1650 kW capacities and it is expected that wind turbines in the range of 2000 to 2500 kW are also being considered for installation in India.

The relationship of wind turbine capacities and rotor diameter with respective heights is given in Figure 2.1.

Figure 2. 1. Relationship between wind turbine capacities and rotor height / diameters

Installed capacities in various countries active in wind power is given in Table 2.2.

Table 2.2 : World wide major installed wind power capacities

| Name of Country | Capacity (MW) |
| --- | --- |
| Germany | 14609 |
| America | 6352 |
| Spain | 6202 |
| Denmark | 3115 |
| India | 2484 |
| Others | 6673 |
| Total installed capacity | 39435 |

It is clear from the above figures that India is one of the leading wind energy producing countries, occupying the 5th position in the world. The state-wise distribution of wind energy capacities installed in India is as follows (Table 2.3). The state of Maharashtra has made progressive advancement in its wind power capacity (Table 2.4).

Table 2.3 : State wise installed wind energy capacities

| Name of State | Capacity (MW) |
| --- | --- |
| Andhra Pradesh | 98 |
| Gujarat | 202 |
| Karnataka | 209 |
| Madhya Pradesh | 26 |
| Rajasthan | 178 |
| Maharashtra | 407 |
| Tamil Nadu | 1361 |
| Others | 3 |
| Total | 2484 |

Table 2.4 : Year wise summary of wind power development in Maharashtra

| Year | Capacity (MW) |
| --- | --- |
| Up to 1995 – 1996 | 2.690 |
| 1996 – 1997 | 2.770 |
| 1997 – 1998 | 0.225 |
| 1998 – 1999 | 19.715 |
| 1999 – 2000 | 50.345 |
| 2000 – 2001 | 117.185 |
| 2001 – 2002 | 206.425 |
| 2002 – 2003 | 00 |
| 2003 – 2004 | 8.25 |
| Total Capacity | 407.605 |

# Identification of feasible sites

Wind measurement have been carried out so far at 72 locations in Maharashtra out of which 28 locations are declared feasible for setting up Wind Power Project in the state (Figure 2.2). The distribution of these sites in various districts is as follows (Table 2.5).

During the year 1996 the first demonstration wind farm in Satara district was established at Chalkewadi with 250 kW wind turbines under sponsorship from MNES. By 1998, the first of privately owned wind turbines were installed at Thosegarh.

Many different sizes of wind turbines were installed in the region.  The sizes ranged from 225 kW to 1250 kW.  But large part of these installations consisted of the 350 kW rating machine.  Table 2.5 gives district wise distribution of wind power in the state of Maharashtra.

The ridge line extending from Vankuswade to Chalkewadi is about 15 km North to south and has a fork like structure on the southern tip.  Compared to this area, wind farm development in other parts of Maharastra is rather small.  Gude Panchgani has about 12.26 MW and Dhalgaon 8.4 MW.  Remaining capacity additions come mainly from Ahmednagar and to a smaller extent from other locations such as Bramhanvel, Amaravati.  These projects have come up during 1999-2002.

Table 2.5 : District wise distribution of wind power sites in Maharashtra

| District | No. of locations |
| --- | --- |
| Kolhapur | 01 |
| Satara | 10 |
| Dhule | 03 |
| Nandurbar | 01 |
| Nashik | 02 |
| Sangli | 03 |
| Ahmednagar | 04 |
| Pune | 01 |
| Beed | 01 |
| Sindhudurg | 01 |
| Aurangabad | 01 |
| Total | 28 |

The wind power development has so far been only concentrated on the plains and ridges of Sahyadri hills in Satara & Sangli districts.  In both of these districts 904 wind turbines are operational contributing 333 MW of Wind Power. The capacity details of these 904 wind turbines are as follows (Table 2.6, Fig. 2.3).

Table 2.6 : Capacities of wind turbines in the state

| Capacity (kW) |  | No. of Wind Turbines |
| --- | --- | --- |
| 225 |  | 47 |
| 230 |  | 144 |
| 250 |  | 12 |
| 350 |  | 556 |
| 500 |  | 21 |
| 600 |  | 88 |
| 750 |  | 29 |
| 1000 |  | 07 |
| Total | 904 |  |

-

Windy sites

ANDHRA PRADESH

ARABIAN SEA

MADHYA PRADESH

KARNATAKA

GUJARAT

Figure 2. 2.  Regional concentration of windy sites identified

REGIONAL CONCENTRATION OF WIND POTENTIAL

THE WINDY SITES ARE CONCENTRATED MOSTLY IN WESTERN MAHARASHTRA AND ON THE HILLS OF SAHYADRI RANGES

(MAHARASHTRA STATE)

Figure 2.3. Percentage of installations in different machine sizes