CONCLUSIONS AND RECOMMENDATIONS

Based on the material available before the Committee and the detailed discussions among its members, the Committee came to the following conclusions:

Conclusions

- The observed decrease in rainfall over the region during the last 5 years is part of large-scale natural variability.  There have been similar instances in the past 100 years.

- Summer monsoon westerly flow extends up to about 5-6 Km vertically. The effect of these wind mills is however limited to roughly 100 m vertically and 200 m horizontally.

- Most of the time the base of low-level clouds is 1500 m above mean sea level and have gigantic dimensions, and the windmills cannot physically disturb such clouds.

- The energy exploited by the windmills is an extremely small fraction of the total wind energy that crosses the wind farm site.

- The windmills, while rotating, cause a small reduction in the                  wind speeds which is made up through mixing with surrounding wind stream in a very short distance behind it.

- Wind farms by their very design do not act as a solid obstruction. Therefore, they do not induce enough vertical velocities to result in any appreciable change in precipitation.

- The flow computations past the windmill blades show no perceptible change in temperature field. Hence, there would be no change in cloud morphology.

- The Committee thus has come to a unanimous conclusion that the windmills have no effect on rainfall.

Keeping in view that the above conclusions are conceptually relevant to the overall monsoon regime, they may be applicable to similar issues related to windmills situated in other parts of the country under the monsoon influence.

# Recommendations

- The Committee feels that the present controversy has arisen due to lack of proper understanding and awareness among the general public, leading to an apparent linkage between the development of the wind farms and the decrease in rainfall over the region.  Therefore, it is recommended that efforts may be made to educate the masses on the environmental friendliness of the wind energy conversion systems.

- As there seems to be an inadequate appreciation in the public mind about the rainfall variability and the robustness of the monsoon phenomenon,      it is desirable that the concerned agencies, universities, NGOs, etc.           may be assigned the task of education and training, necessarily in            the local language.

- All efforts may be made to contain the effects of such controversies            on the development of environment-friendly technologies in the state taking into account concerns of local people.

- The contents of this report may be made available to the public, if the Government so desires.

| - M.P.Ramesh | - K.Sudhakar | - M.Rajeevan |
| --- | --- | --- |
| - Mahesh Vipradas | - A.D.Tillu | - S.V.Singh |

- G.B.Pant

- Pune, 23 July 2004

Acknowledgements

The Committee is grateful to the Honourable Ministers and other Officials of the Industries Energy and Labour Department, Government of Maharashtra for providing them an opportunity                to critically examine this scientifically interesting and                           socio-economically relevant issue of national importance. Committee will like to put on record its appreciation for Shri S.R. Chaudhari, General Manager, Maharashtra Energy Development Agency and his other colleagues for their cooperation in providing the logistic support. We would also like to acknowledge the support provided by the Administration, Library, Information and Publications Division of the Indian Institute of Tropical Meteorology, Pune. The members of public who came to Pune to make presentations before the committee from far off places and the enlightened citizens who gave their inputs to the committee deserve grateful thanks. The media particularly           the local press deserves all appreciation for feedbacks and            balanced reactions. Chairman is personally grateful to his colleague Dr. K. Rupa Kumar for his suggestions and support in preparing             the write up of the report. We thank Dr. (Mrs) P.S. Salvekar and             Mrs A.A. Shiralkar for their help in the interpretation and summarization of the public output particularly those in Marathi.           Shri V.H. Sasane deserves all appreciation for his support in layout and printing of the report.

List of Annexures

Appendix I	: 	Government of Maharashtra order appointing the committee and grant of extension

Appendix II	: 	Newspaper advertisement for public response.

Appendix III	: 	Summary of public response.

Appendix I

Appendix II

Appendix III

Summary of public response

| Sr. No. | Name | Opinion in brief |
| --- | --- | --- |
| 1. | Sh. Chandrashekhar Eknath Barge Advocate, Koregaon Dist. Satara | Position of Chalkewadi, surrounding environment and number of windmills are to be examined properly. Chalkewadi is on the top plateau of Sahyadri and there are about 2000 to 3000 windmills. |
| 2. | Sh. S.M. Ketkar Pune | Wildlife is getting affected due to windmills. |
| 3. | Sh. Prafulla Babar At & Post Mayani Dist. Satara | Direction of windmills is to be changed. At present they are facing towards west so they should be made facing towards east so that moist air will move to the east side and their will be rain. |
| 4. | Dr. L.K. Kshirsagar Pune | Nature beauty will be spoiled because of windmills. There is effect on local climate. There is problem of land conservation. Rainfall was not measured before the work started. |
| 5. | Sh. Samit Moulik Environment Engineer Pune | Needs to study the phenomena through simulation modeling studies. |
| 6. | Prof. Rajendra Kumbhar Karad and Sh. V.A. Kelkar Pune | Windmills on the top of Sahyadri should be stopped at least as an experiment during monsoon season, and further study is to be made. He believes that the windmills are affecting the rainfall. |
| 7. | Sh. Vinayak Kalu Mhetre At & Post Vita, Tal. Khanapur, Dist. Sangli Sh. Harshal Vinayak Mhetre Sh. Chandrakant Dinkar Patil At & Post Waifale, Tal. Tasgaon, Dist. Sangli | Windmills are to be kept off for six months and on for six months. Windmills are not natural, they are man made. So they create disturbances in the natural process. |
| 8. | Sh. Jami Hossain Editor, INWIND Chronicle | No relation of windmills and rain. Windmills are helping to generate the electricity. |
| 9. | Sh. Rahendra V. Kharul Pune | We cannot lead to the conclusion that windmills affects on rain. |
| 10. | Prof. Vijay Janardan Patil Karad, Dist. Satara | There is no relation between windmill and rainfall. |
| 11. | Sh. Mangesh Kashyap and Sh. Prasanna Joshi Pune | Media is disturbing the people in this area. |
| 12. | Dr. S.P. Ranade Sh. S.P. Shinde Smt. Sharvari Kulkarni Pune | Specific Study of this area is required. |
| 13. | Sh. S.S. Shanediwan Namjainagar, Dist. Satara | Windmills should be at sea level and should not be on the top of Sahyadri. |
| 14. | Sh. Arun Ganapati Lad Sh. A.S. Sawat Sh. Vilas M. Jadhav Sh. Hanamant Bapu Lad Sh. Dhananjay Balakrishna Avate Sh. Arjun Sidhu Lad Sh. Arun Pantangrao Lad Sh. Kiran Sambhaji Salunkhe At & Post Kundal, Tal. Palus, Dist. Sangli | There is 40% leakage of electricity. Stoppage to generate electricity during monsoon will not harm. Artificial rain making is very costly. Provide natural rain. |
| 15. | Sh. Hemant Suresh Bhosle, Satara Sh. Mangesh Vijay Yedurkar, Sangli Sh Rushikesh Shilwant, Man Dist. Satara | There is lot of difference between the number of registered windmills and actual available windmills. (They are more than double). Register entry of each and every windmill is required. |
| 16. | Sh. B.H. Shrikanth Pune | Reasons for reduced rainfall are different mainly two are: global warming and reduced green vegetation coverage. Wind power generation indirectly supports to increase the rainfall. |
| 17. | Dr. Bhimaraj Gopal Bhujabal Pune | I am interested to work in this area. At every Taluka place of Satara District, how much rainfall took place in every ‘Nakshatra’ as well as wind direction, temperature and humidity are required to be observed and to be noted. On 28th May in the Nagar district in the region of windmill there was thunderstorm and rain. Many places in Maharashtra there was thunderstorm, hailstorm and heavy rainfall in the last week of May. |
| 18. | Sh. R.S. Nashte Sh. C.J. Aralikatti Pune | Meager electricity is produced out of these windmills. Windmills should be at sea level. Karnataka Govt. has stopped Chitradurg windmill. |
| 19. | Sh. S.B. Ransing (by post card) | Due to rotation of windmill blades clouds get distorted. |
| 20. | Sh. Jayant Vaidya Savewadi, Latur | Due to continuous movement of windmill, clouds get sliced and so clouds go away. To find truth in this statement proper study is needed. In Tamil Nadu 6% increase in rainfall is noticed in the windmill region. In the Maharashtra also Northeast monsoon rainfall is increasing but Southwest monsoon rainfall is decreasing. Therefore we need not demand stopping of windmill till the study is completed. |
| 21. | Sh. Mohan Dasharat Pawar | Instead of talking about remove windmills, we should grow more trees. |
| 22. | Prof. Anand Nivriti Yadav | We should not compare the condition of rain and windmill in the Satara District, and other places. Since 2000, in the east of Sahyadri, year by year rain is reducing drastically. Also there is reduction in the rainy days. |
| 23. | Prof. Vasant Gowarikar and Prof. N.G. Takwale, Pune | (the same opinion). |
| 24. | Sh. Anant Paranjape Karad | Before windmill project started, the rain is already getting reduced and the decrease in rainfall is mainly due to deforestation. |
| 25. | Sh. A.B. Chavhan Phaltan | Rain is reducing due to windmills – this is only misunderstanding and rumour as compared to big and thick clouds and huge Sahyadri mountain, the windmills are like needles. Apart from the 15-20 km surrounding of windmill region other part of Sahyadri like Amboli ghat, Kumbharli ghat, Mahabaleshwar, Bore ghat, Bhimashanker, Kalsubai in all such places also rain is reduced. In the windmill region and surrounding, I myself stayed about 6-6 hours for five different times and personally made observations. Rainfall in both sides of windmill is of same quantity. |
| 26. | Sh. P.B. Shinde Ex-serviceman, Indian Navy, Karad | Height o f the windmill from sea level, height of the clouds moving over the windmills are to be noted carefully and study is to be carried out. Further, observations of temperature, moisture, rainfall at every Taluka place are to be carried out during 10 to 16th day (1st week) of June, July, August and September for the careful study. |
| 27. | Sh. J.J. Godse At. & Post Vaduj, Tal. Khatav, Dist. Satara | Now a days in last few years there is only pre-monsoon shower in Satara District and it is from the east direction. During June to October, video shooting can be made to examine whether clouds are really getting distorted due to windmills or not. |
| 28. | Sh. S.B. Pawar President, Marathi Vigyan Parishad, Kolhapur and Prof. R.V. Bhosale, Senior Prof. retired from PRL and Hon. Prof. Shivaji University, Kolhapur | Further study is needed before any final conclusion. |
| 29. | Sh. Dilip Pawar | Chalkewadi is on the high level plateau on both sides there is valley. On the west, there is Koyna Dam and on the east Satara, Sajjangad. Since height of plateau is too much, the eastern side Pachgani, Sajjangad are also looking at lower levels from the plateau. On this plateau monsoon clouds are at ground level and so the clouds are getting sliced due to windmills. |
| 30. | Sh. Vinayak Anant Pandit Satara | Density of windmills is to be reduced which will actually help in increasing power generation (so that wind strength can be utilized maximum) and also by reducing no. of windmills there will not be any effect on the clouds and so also on the rainfall. |
| 31. | Sh. Kisan Govind More Kari, Satara | Kari village is about 12 kms west of Satara at the foot of Sajjangad. Last 4 years the rainfall is totally reduced. Not even pre-monsoon showers took place. The village land was actually famous for rice, jawar, harbara crops but in the last 4 years farming is not possible and no crop. |
| 32. | Sh. Baban Madhavrao Gaikwad At & Post Arale, Tal. Panhala, Dist. Kolhapur | Instead of calling opinion of different people, one year windmills can be stopped and actual result can be examined. |
| 33. | Sh. Ravindra Baban Lad At & Post Kundal Dist. Sangli | Unknowingly the disturbances are created due to the windmills. As compared to the height of the disturbance (windmill) at 18 times more height, they affect wind (Ref. book by C.G. Bets : Theory of Winds, 1911, 1944). |
| 34. | Sh. Baliram G. Yevle Dy. Sarapanch At & Post Wadi Tal. Khatau, Dist. Satara | In the last 3 to 4 years there is continuous increase in the number of windmills and in this period only rain is found to be reduced. |
| 35. | All India Consumers’ Association Dist. Sangli | Please take the correct decision based on your moral and inner voice. Only for few rich and aggressive people, please don’t do injustice to the simple, poor and ordinary people for the sake of few rich people. |
| 36. | Sh. Gajanan Joshi Pratibhanagar, Satara | Blades are moving due to winds and blades are not creating winds. Original source of energy is in the nature and not manmade. |
| 37. | Sh. G.S. Bengale Gangaves, Kolhapur | In the present rainy season windmills can be kept off for two months and to be examined if there is any difference in rainfall and so common people will be satisfied. |
| 38. | Sh. Laxman Ganapati Supekar Satara | There is no relation between windmill and rainfall. Distance between Sajjangad and Bhor is 30 km. There are no windmills in Bhor. But still rainfall is much less in last few years. Size of windmill is very small and clouds are like hill. How clouds will get disturbed due to windmills. Those people who say that windmills are to be stopped for 4 months, electricity supply in those houses is to be shut down. Clouds, storm, rainfall are natural processes. All these facts you please explain to all the oppositions. |
| 39. | Sh. A.B. Strot Karad | Change the position of windmills and see the result. |
| 40. | Sh. Ashok Suryawanshi Sangli | Due to windmills, direction of clouds gets changed. Therefore, as an experiment only for one month windmills can be stopped. Because of this there may be deficiency in power generation but this loss may be much less than the expenditure required for artificial rain |
| 41. | Writer is farmer of scientific thinking, B.Sc. Tal. Parner, Dist. Ahmednagar | Windmills work due to force of wind. There is no relation between working of windmill and rainfall. Otherwise use of aeroplane or helicopter are also required to stop (which is not the case). We should utilize maximum the natural resources, i.e. wind energy, is to be used with maximum possible. |
| 42. | Retd. Engineers’ Association Sangli Sh. V.M. Marathe Sh. S.S. Garagate Sh. G.R. Kulkarni Sh. R.S. Dewale | Examination of water droplets in the clouds, before the cloud falls on windmill and after crossing the windmill, is required to make in order to verify if there is any effect of windmill on the rainfall. |
| 43. | Sh. Tukaram Dattatrya Bhasme Kolhapur | The thing (or article) which depends on wind, will not oppose wind movement. If due to windmill nearby trees fall down or small size stones (1 kg to 2 kg size) will be thrown away then only there may be some effect on clouds. Otherwise there won’t be any effect. Aeroplanes always move not only in air but also through the clouds. Is there any effect on clouds? In short, there is no effect of windmill on rainfall. |
| 44. | Navyug Mahila Mandal Sangli | Please stop windmills at least for experiment during the period 1 June to 30 September. |
| 45. | Sh. Sudhakar Kedari Ahmednagar | There is no effect of windmill on rainfall. There are different reasons for reduction in the rainfall (Extraction of impure oil reserves should be banned. Frequently occurring earthquakes and the layers of impure irons on the surface of the earth do affect the rainfall). |
| 46. | Sh. Mohan M. Talwalkar Pune (by e-mail) | Windmill does not affect monsoon rain. Windmill is not like fan, driven by outside energy source. Windmill gets itself driven by air current. Windmill is very small and essentially local obstacle. |
| 47. | Advocates of Sangli Bar Association, Sangli (by fax) | They believe that the windmills are absolutely beneficial to its owner’s at the cost of common man. There is no harm to stop the windmills during the monsoon period. |
| 48. | Sh. S.V. Naik Pune | The effect of windmill would be to enhance the possibility of rain. Windmills would not adversely affect the prospects of rain. |
| 49. | Sh. P. Ganguly NCL, Pune (by e-mail) | Monsoon being self organized critical state, at this stage it is not possible to commit the impact of windmill. Further study is very much needed. |
| 50. | Sh. Ravindra B. Khirsagar At & Post Mandhardevi, Tal. Wai, Dist. Satara | There is no relation between monsoon rain and windmill project. The percentage of decrease in rain in last few years may be due to other factors viz. reduction in natural forest and increase in automobiles lead to increase in pollution and hence decrease in rainfall. |
| 51. | Sh S.V. Arumugam Indian Wind Power Association, Chennai | There is no connection between the installation of windmills and the rainfall. |